Produced through free-radical polymerization, LDPE has the most long- and short-chain branching of any form of PE, resulting in its lower density. The branching keeps the molecular chains from packing tightly in its crystaline form, so LDPE has less tensile strength but greater ductility. That exceptional “formability” makes LDPE particularly useful for a range of applications, from rigid products like plastic bottles, buckets and bowls to filmy ones like plastic grocery bags and plastic cling-wrap. Did you have cereal for breakfast? The lining of the milk carton and the plastic bag inside the cereal box were likely made with LDPE.

From a blog at:  www.polymersolutions.com

With its low density and crystalline structure it is flexible but has less tensile strength than the HDPE

### Low-density polyethene

Some polymer chains have side branches. Low-density polyethene (LDPE) is like this.

LDPE

The side branches stop the polymer molecules from lining up regularly. Its structure is not crystalline.

This means that:

- the material has a lower density than HDPE

- the forces of attraction between polymer molecules are weakened

- the material is less strong, and has a lower melting point, than HDPE

BBC Bitesize

On the opposite side of the polymer chain, we have HDPE, which is characterized by minimal branching of the polymer chain. Less branching means those nicely linear molecules pack together well during crystallization, making HDPE much denser and rigid. That added tensile strength means HDPE is the PE of choice for applications that require a bit more “backbone,” such as milk and detergent jugs, garbage cans, water pipes and children’s toys. It’s also one of the reasons why HDPE has largely replaced cardboard as the tube material of choice in making fireworks. An HDPE tube is less prone to shattering if a firework malfunctions, and once the boom inside the tube is exhausted, the HDPE tube is recyclable.

From a blog at:  www.polymersolutions.com

As it says above, more dense and stronger, will withstand much more tensile stress and is therefore better suited to be used for carbonated drinks.

### High-density polyethene

High-density polyethene (HDPE) does not have side branches.

HDPE

The polymer molecules line up regularly to give a crystalline structure.

This means that:

- the material has a higher density than LDPE

- the forces of attraction between polymer molecules are strong

- the material is stronger, and has a higher melting point, than LDPE

BBC Bitesize

Table of common polymers and their properties:

| Polymer | Flexibility | Color | Impact Strength | Chemical resistance | Water Vapor barrier | Gas Barrier | Stress Crack | Impact Resistance | Density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HDPE | Flexible but more rigid than LDPE | Natural color is milky white, semi-translucent depending on density | Good | Good | Good | Poor | Good |  | 0.963 |
| LDPE | Very Flexible | natural milky color, translucent | High | Good | Good | Poor | Good | Good | 0.910–0.940 g/cm3 |
| LLDPE | Very Flexible | natural milky color, translucent | High | Good | Good | Poor | Good | Good | 0.93 |
| Polypropylene | Rigid for containers | Opaque, natural grayish yellow in natural form. |  | Good | Excellent | Poor | Excellent | Excellent | 0.905 g/cm3 |
| Polyvinyl Chloride- PVC | Flexible to Rigid | Transparent to yellowish color in natural state | Low | Good | Good |  | Some |  | 1.35 -1.45 |
| MDPE | Good | Translucent | Good | Excellent |  | Poor | Average | Good | 0.926-0.940 g/cm3 |